Electronic Supplement 1

Accompanying the article: Evolution of a hydrothermal ore-forming system recorded by sulphide mineral chemistry: A case study from the Plaka Pb-Zn-Ag Deposit, Lavrion, Greece

By: Max Frenzel1,2*, Panagiotis Voudouris3, Nigel J Cook4, Cristiana L Ciobanu2, Sarah Gilbert5, Benjamin Wade5

1Helmholtz-Zentrum Dresden-Rossendorf, Institute Freiberg for Resource Technology, Chemnitzer Str. 40, 09599 Freiberg, Germany

2School of Chemical Engineering and Advanced Materials, The University of Adelaide, SA 5005, Australia

3National and Kapodistrian Univ. of Athens, Dept. of Geology and Geoenvironment, University Campus-Zografou 15784 Athens, Greece

4School of Civil, Environmental and Mining Engineering, The University of Adelaide, SA 5005, Australia

5Adelaide Microscopy, The University of Adelaide, SA 5005, Australia

*Corresponding author: m.frenzel@hzdr.de; ORCID: 0000-0001-6625-559X

Contents 									     		Page

Sample list (Table ESM 1.1)								        3

EPMA settings	(in tables)							      	        4

Summary of EPMA data for sphalerite							        6

Estimation of sulfur fugacity from sphalerite compositions			        	        7

References cited in this ESM				       				       12

Table EMS1.1 – Sample overview

| Sample | Mineralisation style | Area | Location |  | Sulfide mineralogya |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Lat (°N) | Long (°E) |  |
| 46 | Carbonate replacement | Villia | 37° 46.049´ | 24° 1.819´ | Sp >> Py > Gn >> Ccp |
| 65 | Carbonate replacement | Villia | 37° 45.927´ | 24° 2.140´ | Sp >> Gn > Py >> Ccp > Po |
| 38 | Carbonate replacement | Near Vein 80 | 37° 45.666´ | 24° 2.028´ | Ccp >> Py > Sp > Gn > Po |
| FL8012 | Vein | Vein 80 | 37° 45.558´ | 24° 2.133´ | Py > Sp ~ Ccp > Ttr > Gn ~ Po |
| FL8015 | Vein/ Carbonate replacement | Vein 80 | 37° 45.558´ | 24° 2.133´ | Sp > Ccp > Gn >> Ttr > Py |
| PLA-7 | Vein | Vein 80 | 37° 45.655´ | 24° 1.700´ | Gn >> Sp >> Ttr > Apy |

aMineral abbreviations following Whitney and Evans (2010): Ccp – chalcopyrite, Apy – arsenopyrite, Gn – galena, Po – pyrrhotite, Py – pyrite, Sp – sphalerite, Ttr – tetrahedrite

Note: Non-sulfide gangue minerals are mostly quartz, siderite and calcite

Table ESM1.2 – EPMA setup for spot analyses of sphalerite

| Element and Line | Diffracting Crystal (Sp#) | Background type/fit | kV/nA/spot size(µm) | Peak Count Time | Bkgd Count Times |  | # bkgd points acquired (Lo/Hi) | Standards* |  | Count Detection limit, 99% confidence (wt.%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Lo | Hi |  | Primary Standard | Interference Standards |  |
| S Ka | LPET (1) | Multipoint | 20/30/3 | 15 | 5 | 5 | 2/2 | 564 | 582 | 0.01 |
| Cd La | LPET (1) | Multipoint | 20/30/3 | 15 | 5 | 5 | 0/4 | 561 | 566,560,568 | 0.03 |
| Pb Ma | LPET (1) | Multipoint | 20/30/3 | 15 | 5 | 5 | 4/3 | 566 | 556,562 | 0.03 |
| Na Ka | TAP (2) | Multipoint | 20/30/3 | 15 | 5 | 5 | 2/2 | 501 | 564 | 0.04 |
| As La | TAP (2) | Multipoint | 20/30/3 | 15 | 5 | 5 | 2/2 | 562 | 556 | 0.04 |
| Se La | TAP (2) | Multipoint | 20/30/3 | 15 | 5 | 5 | 2/3 | 568 | 562 | 0.03 |
| Fe Ka | LLIF (3) | Multipoint | 20/30/3 | 15 | 5 | 5 | 2/2 | 556 | 566 | 0.01 |
| Cu Ka | LLIF (3) | 2-pt linear | 20/30/3 | 15 | 5 | 5 | 1/1 | 556 |  | 0.02 |
| Mn Ka | LLIF (3) | Multipoint | 20/30/3 | 15 | 5 | 5 | 2/2 | 557 | 562 | 0.01 |
| Ga Ka | LLIF (3) | Multipoint | 20/30/3 | 15 | 5 | 5 | 3/2 | 562 | 564 | 0.02 |
| Ge Ka | LLIF (3) | Multipoint | 20/30/3 | 15 | 5 | 5 | 2/2 | 592 | 564 | 0.04 |
| Ag La | LPET (4) | Multipoint | 20/30/3 | 15 | 5 | 5 | 1/2 | 560 | 556,557 | 0.04 |
| Sn La | LPET (4) | Multipoint | 20/30/3 | 15 | 5 | 5 | 2/2 | 573 | 602,565 | 0.02 |
| In La | LPET (4) | Multipoint | 20/30/3 | 15 | 5 | 5 | 5/0 | 605 | 561,565 | 0.02 |
| Zn Ka | LPET (5) | Multipoint | 20/30/3 | 15 | 5 | 5 | 2/2 | 564 |  | 0.02 |
| Hg La | LPET (5) | Multipoint | 20/30/3 | 15 | 5 | 5 | 3/3 | 565 | 564 | 0.07 |
| Ni Ka | LPET (5) | 2-pt linear | 20/30/3 | 15 | 5 | 5 | 1/1 | 536 |  | 0.01 |
| Co Ka | LPET (5) | Multipoint | 20/30/3 | 15 | 5 | 5 | 2/2 | 602 |  | 0.01 |
| * Standard # refers to internal database. Full list of standards in Table ESM 1.4 |  |  |  |  |  |  |  |  |  |  |

Table ESM1.3 – Interference corrections for spot analyses of sphalerite

| Element and Line | Overlapping element and order/interference correction standard |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Interfering line | Int. std* | Interfering line | Int. std* | Interfering line | Int. std* |
| S Ka | Pb Ma (IV) | 582 |  |  |  |  |
| Cd La | Pb Ma (IV) | 566 | Ag La (I) | 560 | Se La (IV) | 568 |
| Pb Ma | Fe Ka (III) | 556 | As La (V) | 562 |  |  |
| Na Ka | Zn Ka (I) | 564 |  |  |  |  |
| As La | Fe Ka (V) | 556 |  |  |  |  |
| Se La | As La (I) | 562 |  |  |  |  |
| Fe Ka | Pb Ma (II) | 566 |  |  |  |  |
| Cu Ka |  |  |  |  |  |  |
| Mn Ka | As La (II) | 562 |  |  |  |  |
| Ga Ka | Zn Ka (I) | 564 |  |  |  |  |
| Ge Ka | Zn Ka (I) | 564 |  |  |  |  |
| Ag La | Cu Ka (III) | 556 | Mn Ka (II) | 557 |  |  |
| Sn La | Co Ka (II) | 602 | Hg La (IV) | 565 |  |  |
| In La | Cd La (I) | 561 | Hg La (III) | 565 |  |  |
| Zn Ka |  |  |  |  |  |  |
| Hg La | Zn Ka (I) | 564 |  |  |  |  |
| Ni Ka |  |  |  |  |  |  |
| Co Ka |  |  |  |  |  |  |

* Standard # refers to internal database. Full list of standards in Table ESM 1.4

Table ESM1.4 – Standard information

| Reference # | Mineral composition | Natural/Synthetic | Manufacturer |
| --- | --- | --- | --- |
| 501 | Albite | Natural | Astimex |
| 556 | Chalcopyrite | Natural | P&H and Associates |
| 557 | Rhodonite | Natural | P&H and Associates |
| 560 | Silver Telluride | Synthetic | P&H and Associates |
| 561 | Greenockite | Synthetic | P&H and Associates |
| 562 | Gallium Aresenide | Synthetic | P&H and Associates |
| 564 | Sphalerite | Natural | P&H and Associates |
| 565 | Cinnabar | Natural | P&H and Associates |
| 566 | Galena | Natural | P&H and Associates |
| 568 | Bismuth Selenide | Synthetic | P&H and Associates |
| 573 | Cassiterite | Natural | P&H and Associates |
| 582 | Lead | Synthetic | Astimex |
| 592 | Germanium | Synthetic | Astimex |
| 602 | Cobalt | Synthetic | Astimex |
| 605 | Indium | Synthetic | Astimex |

Table ESM1.5 – Summary of EPMA analyses of sphalerite (arithmetic means)

| Sample | Generation | n | Fe (wt.%) | Mn (wt.%) | Cd (wt.%) | Cu (wt.%) | Co (wt.%) | In (wt.%) | Zn (wt.%) | S (wt.%) | Sum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 38 | I | 14 | 6.7 | 0.03 | 0.30 | 0.16 | 0.05 | <0.02 | 59.4 | 33.0 | 99.7 |
|  | II | 1 | 0.28 | <0.02 | 0.22 | 0.03 | <0.01 | <0.02 | 66.8 | 32.7 | 100.1 |
| 46 | I | 14 | 12.0 | 0.45 | 0.15 | <0.02 | 0.01 | <0.02 | 53.5 | 33.7 | 99.8 |
| 65 | I | 15 | 12.1 | 0.46 | 0.19 | 0.06 | 0.02 | <0.02 | 53.2 | 33.0 | 99.1 |
| FL8012 | I | 8 | 12.1 | 0.28 | 0.22 | 0.04 | 0.07 | 0.04 | 53.4 | 33.3 | 99.5 |
|  | II | 9 | 0.98 | <0.02 | 0.18 | 0.05 | <0.02 | <0.02 | 65.8 | 32.8 | 99.9 |
| FL8015 | I | 14 | 6.6 | 0.22 | 0.22 | 2.1 | 0.07 | 0.06 | 57.5 | 32.8 | 99.5 |
|  | II | 9 | 1.5 | 0.03 | 0.15 | 0.09 | <0.01 | 0.03 | 65.3 | 32.4 | 99.5 |
| PLA-7 | II | 18 | 1.5 | <0.02 | 0.36 | 0.07 | <0.01 | <0.02 | 65.2 | 32.5 | 99.7 |

Estimation of sulfur fugacity from sphalerite composition

The purpose of this section is to explain in detail the data, procedures and assumptions used in estimating the sulfur fugacity values presented in Table 2 in the main text. The necessity for such a separate description arises from the fact that no adequate up-to-date model of the relevant experimental data is currently available in the literature. It was therefore necessary to refit the available data to ensure robust and reliable predictions, particularly for sphalerite II, whose formation temperature generally falls outside of the range covered in experimental research.

Background

Barton and Toulmin (1966) first presented detailed experimental evidence to show that the Fe-content of sphalerite in equilibrium with pyrite and/or pyrrhotite is sensitive to both temperature and sulfur fugacity. These authors worked at relatively high temperatures, between 550 – 1,100°C. Later work by Scott and Barnes (1971), Scott and Kissin (1973), and Lusk and Calder (2004) extended the range of experimental data down to lower temperatures around 280°C, which are more relevant for the study of hydrothermal mineral deposits.

Theory

The dependence of the Fe content in sphalerite on sulfur fugacity is a consequence of the different molar Fe:S ratios prevailing in sphalerite and the Fe-sulfide minerals. While the Fe:S ratio in sphalerite is 1:1, it is 1:2 in pyrite, and (1-x):1 in pyrrhotite. Pyrite is the most common Fe sulphide encountered in hydrothermal base-metal deposits, including Plaka, and the following discussion therefore focusses on this mineral.

The incorporation of Fe from pyrite into sphalerite is best described by the reaction:

2 FeS2 py↔2 FeS sp+S2        (ESM1.1)

where FeS sp denotes the FeS-component in sphalerite. The equilibrium constant for this reaction is:

K=FeS sp2∙[S2][FeS2]        (ESM1.2)

where the square brackets denote chemical activities.

Since natural pyrite is generally relatively pure FeS2 (< 5 wt.% total impurities, cf. Table 4 for Plaka pyrite), it is reasonable to assume that:

FeS2≈ 1        (ESM1.3)

By definition, we also have:

S2= fS2p0       (ESM1.4)

where p0 is a reference pressure, usually taken as 1 bar.

Taking logs of both sides of (ESM1.2), substituting in relations (ESM1.3) and (ESM1.4), and re-arranging for logFeS sp, we have:

logFeS sp=12logK-logp0-logfS2       (ESM1.5)

Finally, making the substitution:

logK= -∆GRT       (ESM1.6)

where ∆G is the Gibbs free energy of formation, T is temperature and R is the gas constant, gives:

logFeS sp=-12( ∆GRT+logp0+ logfS2)       (ESM1.7)

This clearly shows the close relationship between temperature, fS2 and the activity of the FeS-component in sphalerite. As demonstrated by Barton and Toulmin (1966), the activity of FeS is closely related to the Fe-content.

Because pyrite is the most common Fe-sulfide mineral associated with sphalerite in hydrothermal mineral deposits, relationship (ESM1.7) forms the general basis for the estimation of fS2 values from sphalerite compositions (cf. Barton and Toulmin 1966, Lusk and Calder 2004), provided the formation temperature is known. Specifically, we have:

logfS2=-2logFeS sp+ ∆GRT+logp0= -2logFeS sp+ ∆HRT-∆SR+logp0      (ESM1.8)

Where ∆H and ∆S are the enthalpy and entropy of formation, respectively. It was stated earlier that no adequate up-to-date model is available for the estimation of fS2 from sphalerite composition. The reason for this is that Lusk and Calder (2004) who produced the most recent compilation of old and new experimental data fitted a model of the form:

logfS2=A+BT+(C-DT)(mol.% FeS)       (ESM1.9)

Comparing this with the theoretical expectation in (ESM1.8), it does not seem to be an adequate representation of the expected relationship. Replacing the dependence on logFeS sp with a temperature-dependent linear relationship on Fe-concentration will only work reasonably well for high Fe-contents and in the temperature range of the available data, but does not allow for confident extrapolation beyond the limits of the experimental data. Therefore, this fit is not expected to behave well when applied to any natural sphalerite having formed either below temperatures of ~300°C or having an Fe-content below ~1 wt.%. This is graphically illustrated in Figure ESM1.1, which compares the new model used in the present article (Fig. ESM1.1A) with that of Lusk and Calder (2004) (Fig. EMS1.1B). It shows that the fit presented by Lusk and Calder (2004) diverges considerably from our theoretically validated model across a broad range of temperatures and Fe-contents in sphalerite. Only their 20 mol.% FeS-isoline is reasonably close to our model.

Data

The experimental data used here for FeS contents in sphalerite, formation temperatures and sulfur fugacities is a compilation from the same sources as that used in Lusk and Calder (2004). It is summarised in Table ESM1.6. Only data for temperatures up to ~ 700°C, and with CuS contents in sphalerite of less than 0.5 mol.% were considered. One low-temperature data-point from the study of Scott and Kissin (1973) was added to ensure greater robustness in fitting the low-T data. This was completed with suitable fS2 data from Lusk and Bray (2002). Furthermore, we made use of the data on

Fig. ESM1.1: Comparison between models of Fe-content in sphalerite as a function of fS2 and temperature: A) theoretically validated model fitted in this article (similar to the model used by Barton and Toulmin, 1966); B) empirical model fitted by Lusk and Calder (2004). The inability of the Lusk and Calder (2004) model to realistically depict the behaviour for Fe-concentrations < 1 mol% FeS (all expected to lie on virtually the same isoline), as well as at temperatures below ~ 280°C is due to its inadequate mathematical form, as discussed under “Theory”. Mineral reaction lines and sulfidation-state fields after Einaudi et al. (2003). Mineral abbreviations identical to those used in Fig. 4 in the main text.

Table ESM1.6 – Experimental data on Fe-contents in sphalerite in sp + py ± po assemblages

| Temperature (°C) | FeS (mol.%) | CuS (mol.%) | log10fS2 | Mineral assemblage | Reference(s) |
| --- | --- | --- | --- | --- | --- |
| 519 | 21.8 | 0 | -3.85 | sp + py + po | Lusk & Calder (2004) |
| 451 | 21.8 | 0 | -5.62 | sp + py + po | Lusk & Calder (2004) |
| 435 | 21.7 | 0 | -6.08 | sp + py + po | Lusk & Calder (2004) |
| 410 | 22.2 | 0 | -6.86 | sp + py + po | Lusk & Calder (2004) |
| 369 | 22.7 | 0 | -8.25 | sp + py + po | Lusk & Calder (2004) |
| 372 | 22.7 | 0.3 | -8.1 | sp + py + cp + po | Lusk & Calder (2004) |
| 308 | 20.5 | 0 | -10.3 | sp + py + po | Scott & Barnes (1971) |
| 350 | 20.5 | 0 | -8.68 | sp + py + po | Scott & Barnes (1971) |
| 400 | 20.5 | 0 | -7.02 | sp + py + po | Scott & Barnes (1971) |
| 450 | 20.5 | 0 | -5.59 | sp + py + po | Scott & Barnes (1971) |
| 500 | 20.5 | 0 | -4.39 | sp + py + po | Scott & Barnes (1971) |
| 550 | 20.5 | 0 | -3.33 | sp + py + po | Scott & Barnes (1971) |
| 722 | 11.8 | 0 | 0.72 | sp + py + L | Barton & Toulmin (1966) |
| 700 | 16.1 | 0 | 0 | sp + po + py | Barton & Toulmin (1966) |
| 700 | 8.5 | 0 | 0.58 | sp + py + L | Barton & Toulmin (1966) |
| 690 | 6.7 | 0 | 0.52 | sp + py + L | Barton & Toulmin (1966) |
| 700 | 17.9 | 0 | 0 | sp + po + py | Barton & Toulmin (1966) |
| 640 | 17.9 | 0 | -1.24 | sp + po + py | Barton & Toulmin (1966) |
| 650 | 17.3 | 0 | -0.94 | sp + po + py | Barton & Toulmin (1966) |
| 636 | 18.6 | 0 | -1.22 | sp + po + py | Barton & Toulmin (1966) |
| 636 | 18.1 | 0 | -1.22 | sp + po + py | Barton & Toulmin (1966) |
| 600 | 18.3 | 0 | -1.97 | sp + po + py | Barton & Toulmin (1966) |
| 600 | 16.8 | 0 | -1.97 | sp + po + py | Barton & Toulmin (1966) |
| 580 | 0.9 | 0 | -0.26 | sp + py + L | Barton & Toulmin (1966) |
| 580 | 20.8 | 0 | -2.41 | sp + po + py | Barton & Toulmin (1966) |
| 580 | 17.8 | 0 | -2.41 | sp + po + py + mag | Barton & Toulmin (1966) |
| 555 | 19 | 0 | -2.98 | sp + po + py + mag | Barton & Toulmin (1966) |
| 555 | 19.7 | 0 | -2.98 | sp + po + py + mag | Barton & Toulmin (1966) |
| 277 | 20.6 | 0 | -11.95 | sp + py + po | FeS-content from Scott & Kissin (1973); fS2 from Lusk & Bray (2002) |

FeS-contents and -activities in sphalerite provided in Table 3 of Barton and Toulmin (1966), as discussed further below, as well as the data on the temperature-dependence of the maximum solubility of FeS in sphalerite produced by Balabin and Urusov (1995).

Fitting procedure

For the present model, we used a two-step fitting procedure in which we first calculated the temperature- and pressure-corrected activity of the FeS-component in sphalerite, and subsequently used this to fit a parameterised version of relation (ESM1.8). All calculations were performed using the R software package.

Starting with the FeS-activity data from Barton and Toulmin (1966) (their Table 3), we fitted a model of the form

log10FeS sp=alog10FeS+b      (ESM1.10)

where FeS denotes the Fe-content in sphalerite, expressed in mol.% FeS. This form was chosen to capture the apparent non-linear relationship between the two quantities. It provides an excellent fit to the data, with R2=0.984. The best estimates for the fitting parameters are: a=0.80±0.02 and b= -1.38±0.03.

Both a temperature- and pressure-adjustment were then incorporated by scaling this relationship in such a way that [FeS sp]=1 when FeS=FeSmax, where FeSmax is the temperature- and pressure- dependent maximum solubility of FeS in sphalerite (in mol.% FeS) when in contact with metallic Fe and troilite (cf. Barton and Toulmin 1966; Balabin and Urusov 1995). That is, the final model for [FeS sp] is:

log10FeS sp=alog10(53.1∙FeSFeSmax) +b      (ESM1.11)

with parameters a and b the same as in (ESM1.10), and the following equation for FeSmax:

FeSmax=44.09+0.0125∙T-3600T∙p       (ESM1.12)

where T is temperature in Kelvin, and p is pressure in kbar. Relation (ESM1.12) is a composite of the temperature dependence determined by Balabin and Urusov (1995), and the pressure dependence from Barton and Toulmin (1966).

Relationship (ESM1.11) was then used to re-calculate FeS-activity values for the experimental data in Table ESM1.6. Having completed the dataset in this way, we fitted a relationship of the form:

log10fS2=-2log10FeS sp+ AT+B      (ESM1.13)

to the data. This provides an excellent fit with R2= 0.999. The standard deviation of the residual values is only 0.13 units of log10fS2. The best estimates for the fitting parameters are: A= -1.50±0.02×104 and B=14.5±0.3.

With this revised fit, the expected estimation error on log10fS2 even at a temperature as low as ~100°C is only ±0.35 units (95% confidence). Therefore, it is expected to yield acceptable results over the entire range of relevant ore-forming conditions for hydrothermal sphalerite, i.e., from <100 – 500°C.

A further potential limitation of this model concerns Cu contents in sphalerite. Lusk and Calder (2004) showed that the presence of Cu may affect the incorporation of Fe and thus the estimation of sulfur fugacities. Therefore, it is important to assess the exact effects of Cu incorporation on estimated sulfur fugacities. We did this by first calculating the deviations of actual sulfur fugacities in Cu-containing charges investigated by Lusk and Calder (2004) from the values expected from Fe contents alone:

dfS2Cu=log10fS2Cu-log10fS2eqn. ESM1.13         (ESM1.14)

where log10fS2eqn. ESM1.13 is the estimated log10fS2 value from equation (ESM1.13). We then assessed the relationship between dfS2 and Cu concentration by fitting the following model:

dfS2Cu=C∙log10CuS+D      (ESM1.15)

Where C and D are constants, and CuS is the concentration of Cu in sphalerite, expressed in mol.% CuS. This produced a reasonable fit, with an adjusted R2 of 0.75. The best estimates of the fitted parameters are: C = 3.4±1.4 and D =1.1±0.4. Using these results, appreciable deviations from Cu-free experiments (more than 0.3 units of log10fS2) only occur above Cu contents of about 0.6 mol.% CuS, or ~0.6 wt.% Cu. For sphalerites with Cu concentrations below this limit, no correction should be applied. For sphalerites with higher Cu contents, however, correction should be made according to equation (ESM1.15).

Estimation of sulfur fugacities at Plaka

To estimate the sulfur fugacities prevailing at the Plaka deposit, we first recalculated the measured Fe-contents in sphalerite to mol.% FeS using the Fe/Zn ratios determined by LA-ICP-MS. We then used these concentrations to estimate FeS-activities from equation (ESM1.11), using a formation pressure of 0.2 ± 0.1 kbar (cf. Voudouris et al. 2008) and estimated formation temperatures from the GGIMFis geothermometer. The resultant FeS-activity values were then combined with GGIMFis temperatures to calculate fS2, including uncertainties, from equation (ESM1.13). Results are summarised in Table 2. The major source of the external uncertainties reported in Table 2 is the uncertainty in absolute formation temperatures. This causes an uncertainty of about ±2 units of log10fS2 in all cases. All other uncertainties together, including the calibration uncertainty of relation (ESM1.13), as well as variations in sphalerite composition, amount to no more than ±0.5 units of log10fS2.

No correction was made for the effect of Cu concentrations (cf. eqn. (ESM1.15) above) since these were below 0.6 wt.% in most cases. The only exception is sp I from sample FL8015. Here, the high Cu contents appear to be largely due to abundant inclusions of chalcopyrite in the sphalerite. Such inclusions do not affect the applicability of relationship (ESM1.13), and therefore, no correction was applied to this sphalerite either.

By estimating log10fS2 as described above we made one major assumption: namely, that Fe-contents in sphalerite are buffered by pyrite in all cases. Pyrite is present in most samples and is generally an abundant and widely distributed mineral at Plaka. It occurs throughout the complete paragenetic sequence (Voudouris et al. 2008). It is always present with sp I in our samples, but not generally with sp II. Nevertheless, its abundance means that the fluids corresponding to the precipitation of sp II would have been in contact with pyrite on their way to, and generally in close proximity to, the site of mineralisation, even if pyrite and sphalerite did not precipitate together. This means that pyrite-buffering is a reasonable assumption in the present case. The same working principle was adopted by Barton et al. (1977) for their classic study of the Creede district, Colorado.

The alternative to pyrite buffering, i.e., [FeS2]≈1, would be that pyrite is undersaturated, i.e. FeS2< 1. In this case, log10fS2 values estimated using equations (ESM1.10) to (ESM1.13) would be higher than the true values (cf. eq. (ESM1.2)). This difference would be equal to -log10FeS2. Thus, large changes in FeS2, i.e., by orders of magnitude, would be necessary to produce appreciable changes in estimated sulphur fugacities. Such changes are unlikely to occur in the present case, given the aforementioned constraints.

References cited in this ESM

Balabin AI, Urusov VS (1995) Recalibration of the sphalerite cosmobarometer: Experimental and	theoretical treatment. Geochim Cosmochim Ac 59:1401–1410.

Barton PB, Bethke PM, Roedder E (1977) Environment of ore deposition in the Creede mining district,	San Juan Mountains, Colorado: Part III, Progress toward interpretation of the chemistry of the	ore-forming fluid for the OH Vein. Econ Geol 72:1-24.

Barton PB, Toulmin P III (1966) Phase relations involving sphalerite in the Fe-Zn-S system. Econ Geol	61:815–849.

Einaudi MT, Hedenquist JW, Inan EE (2003) Sulfidation state of fluids in active and extinct hydrothermal	systems: transition from porphyry to epithermal environments. In: Simmons SF and Graham I	(eds) Volcanic, geothermal, and ore-forming fluids: rulers and witnesses of processes within	the Earth, SEG Special Publication, vol. 10, pp 285-313

Lusk J, Bray MD (2002) Phase relations and the electrochemical determination of sulfur fugacity for	selected reactions in the Cu-Fe-S and Fe-S systems at 1 bar and temperatures between 185	and 460°C. Chem Geol 192:227–248.

Lusk J, Calder BOE (2004) The composition of sphalerite and associated sulfides in reactions of the Cu-	Fe-Zn-S, Fe-Zn-S and Cu-Fe-S systems at 1 bar and temperatures between 250 and 535°C. Chem	Geol 203:319–345.

Scott SD, Barnes HL (1971) Sphalerite geothermometry and geobarometry. Econ Geol 66:653–669.

Scott SD, Kissin SA (1973) Sphalerite composition in the Zn-Fe-S system below 300°C. Econ Geol	68:475–479.

Voudouris P, Melfos V, Spry PG, Bonsall TA, Tarkian M, Economou-Eliopoulos M (2008) Mineralogical	and fluid inclusion constraints on the evolution of the Plaka intrusion-related ore system,	Lavrion, Greece. Miner Petrol 93:79-110